Secure Wireless Communication using nRF24L01 and XTEA

Overview

Designed and implemented a secure wireless communication system between two Arduino nodes using nRF24L01 transceivers. The project employs the XTEA (eXtended Tiny Encryption Algorithm) to encrypt messages before transmission and decrypt them upon reception, ensuring confidentiality of wireless data exchanged over a 2.4 GHz link.

Features

- Peer-to-peer wireless communication

- 128-bit XTEA encryption

- Real-time encrypted messaging

- Bidirectional communication support

- Hexadecimal ciphertext monitoring for debugging

- Lightweight implementation suitable for resource-constrained microcontrollers

- Message padding and block-based encryption

Hardware Requirements

- 2 × Arduino Uno/Nano

- 2 × nRF24L01 Wireless Modules

- Jumper wires

- Power supply

Software Requirements

- Arduino IDE

- RF24 Library

- SPI Library

- nRF24L01 Library

System Architecture

Sender Arduino
      │
      ▼
Plain Text Message
      │
      ▼
XTEA Encryption
      │
      ▼
nRF24L01 Transmission
      │
      ▼
Wireless Channel
      │
      ▼
nRF24L01 Reception
      │
      ▼
XTEA Decryption
      │
      ▼
Recovered Plain Text

Implementation Details

Wireless Layer

- Utilized nRF24L01 transceivers operating at 2.4 GHz.

- Configured dedicated transmit and receive pipes for each node.

- Supported bidirectional communication using separate addresses.

Encryption Layer

- Implemented XTEA block cipher with:

  - 128-bit secret key

  - 64-bit block size

  - 32 encryption rounds

- Messages are divided into 8-byte blocks and encrypted before transmission.

Data Processing

- Plaintext converted into fixed-size encrypted packets.

- Zero-padding applied for messages shorter than 32 bytes.

- Received packets decrypted and reconstructed into readable text.

Key Skills Demonstrated

- Embedded Systems Development

- Secure Communication Protocols

- Wireless Networking

- Cryptography Fundamentals

- Arduino Programming

- SPI Communication

- Real-Time Data Transmission

- Debugging and Testing

Project Outcomes

- Successfully established encrypted wireless communication between two microcontroller nodes.

- Prevented direct interpretation of intercepted radio packets by encrypting payloads.

- Demonstrated integration of cryptographic algorithms with low-power embedded hardware.

Future Enhancements

- AES-128 encryption support

- Dynamic key exchange mechanism

- Message authentication (HMAC)

- Replay attack protection

- Multi-node secure network support

- OLED display integration for standalone messaging

Technologies Used

- Arduino C/C++

- XTEA Cryptography

- nRF24L01 RF Modules

- SPI Protocol

- RF24 Library